NPC(N₂PurgeController)ONLINE

COMMUNICATIONS SPECIFICATIONS

Version 1.0.0

Date	September 11, 2024

Serial No.	0

# Contents

List of Tables	iv

1	INTRODUCTION	5

1.1	Purpose	5

1.2	Referenced Documentation	5

2	CONFORMANCE TO  SEMI STANDARD	6

2.1	SECS Communication	6

2.2	Equipment Model	7

2.2.1	State Model List	8

3	DATA ITEMS	9

3.1	SECS Data Items	10

3.2	Table of Variables	14

3.2.1	VID Numbering	14

3.2.2	NPC variables	14

4	SECS MESSAGES	16

4.1	Message List	16

5	EVENTS	21

5.1	EVENT and EVENT REPORT	21

5.1.1	CEID Numbering	21

5.1.2	NPC Events	22

1）	Port Status Transition Events	22

2）	Alarm Related Events	22

5.1.3	NPC Event Reports	23

6	SCENARIO	24

6.1	COMMON SCENARIO	24

6.1.1	ACTIVATION FROM THE AMHS  (from EQUIPMENT OFFLINE)	25

6.1.2	ACTIVATION FROM THE HOST (ONLINE REMOTE)	26

6.2	NPC SCENARIO	27

6.2.1	PURGE UNIT ERROR OCCURS	27

6.2.2	PURGE UNIT ERROR RELEASE	27

7	Appendix	28

7.1	Using VID when synchronize	28

7.1.1	NPC	28

7.2	Revision History	28

# List of Tables

Table 1	- SECS Communication SEMI Standards	6

Table 2	- Equipment Model SEMI Standards	7

Table 3	- Gem Compliant Items in the Stocker SEM	7

Table 4	- State Model List	8

Table 5	- SECS Data Items Used by the NPC	10

Table 6	- VID Numbering	14

Table 7	- Variables used by the NPC	15

Table 8	- SECS Messages Used by the NPC	17

Table 13	- Basic Event Report Format	23

INTRODUCTION

Purpose

The purpose of this document is to define the communication specifications to be used in online communication with the NPC.

Referenced Documentation

SEMI Doc E5－	EQUIPMENT COMMUNICATIONS STANDARD 2 
MESSAGE CONTENT(SECS-II)

SEMI Doc E30－　　GENERIC MODEL FOR COMMUNICATIONS AND 
CONTROL OF　MANUFACTURING EQUIPMENT (GEM)

SEMI Doc E37－	HIGH-SPEED SECS MESSAGE SERVICES (HSMS) 
GENERIC SERVICES

SEMI Doc E37.1－	HIGH-SPEED SECS MESSAGE SERVICES 
SINGLE-SESSION MODE　(HSMS-SS)

SEMI Doc E88-302@2002－	Specification for AMHS Storage SE

CONFORMANCE TO
 SEMI STANDARD

SECS Communication

Communication between the NPC and host system must conform to the following SEMI standards.

Table 1	- SECS Communication SEMI Standards

| No | SEMI STANDARD | Description |
| --- | --- | --- |
| 1 | SEMI Doc E5 | SEMI EQUIPMENT COMMUNICATIONS STANDARD 2 MESSAGE CONTENT(SECS-II) |
| 2 | SEMI Doc E37 | HIGH-SPEED SECS MESSAGE SERVICES (HSMS) GENERIC SERVICES |
| 3 | SEMI Doc E37.1 | HIGH-SPEED SECS MESSAGE SERVICES SINGLE-SESSION MODE (HSMS-SS) |

Equipment Model

The NPC is equipped with equipment models compliant to the following SEMI standards.

Table 2	- Equipment Model SEMI Standards

| No | SEMI STANDARD | Description |
| --- | --- | --- |
| 1 | SEMI Doc E30 | The following sections from GENERIC MODEL FOR COMMUNICATIONS AND CONTROL OF MANUFACTURING EQUIPMENT (GEM): Sections 1, 2 & 3: all Section 4: 4.1 ,4.2.1 ,4.2.2 ,4.2.5 ,4.2.6 ,4.3 4.5 ,4.7 ,4.8 ,4.9 ,4.10 ,4.12 Section 8: all |

Table 3	- Gem Compliant Items in the Stocker SEM

| No | GEM Basic Condition |  | No | Additional Capabilities | 要否 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | status model |  | 1 | communications establishment | ○ |
| 2 | equipment process status |  | 2 | dynamic event report setting modifying | ○ |
| 3 | host initiated S1, F13/F14 cenarios |  | 3 | variable data collection | ○ |
| 4 | event notification |  | 4 | trace data collection | × |
| 5 | online confirmation |  | 5 | status data collection | ○ |
| 6 | error messages |  | 6 | alarm control | ○ |
| 7 | control (operator activated) |  | 7 | remote control | × |
| 8 | documentation |  | 8 | equipment constants | ○ |
|  |  |  | 9 | process program control | × |
|  |  |  | 10 | material movement | × |
|  |  |  | 11 | equipment terminal service | ○ |
|  |  |  | 12 | clock | ○ |
|  |  |  | 13 | limit monitoring | × |
|  |  |  | 14 | spurring | × |
|  |  |  | 15 | control (host activated) | ○ |

（要否凡例）　○： Necessary　　×： Not necessary

State Model List

The state models to be included in the SC are described in both SEM and GEM. The following is a list of those state models:

Table 4	- State Model List

| No | Model Name | Reference |
| --- | --- | --- |
| 1 | Communication State Model | GEM 3.2 Communication State Model |
| 2 | Controller State Model | GEM3.3 Controller State Model |

The NPC also contains an HSMS-SS state model. (Refer to section 5 of SEMI E37.1 HSMSSS).

DATA ITEMS

This chapter explains the SECS message data items used by the NPC.

SECS Data Items

The following chart contains the SECS message data items used by the NPC. The features of the chart are as follows:

< Data in the chart >

Name: The name of the data item.

Format: The format of the data item value. However, "Ux" is sent as "U2" from the host.

SIZE: The size of the data item value.

EXPLANATION: An explanation of the data item and/or the range of 
the data.

Table 5	- SECS Data Items Used by the NPC

| No | Name | Format | Size | Explanation |
| --- | --- | --- | --- | --- |
| 1 | ACKC5 | B | 1 | 0 :ACK ≦0 :error, could not ACK 1～63 :hold |
| 2 | ACKC6 | B | 1 | 0 :ACK ≦0 :error, could not ACK 1～63 :hold |
| 3 | ACKC10 | B | 1 | - :ACK: 1 : message not displayed 2 :terminal PC is unavailable 3～63 :hold |
| 4 | ALCD | B | 1 | Bit8 = 1 : generated alarm status Bit8 = 0 : clear alarm status Bit1～7 :not used |
| 5 | ALED | B | 1 | Bit8 = 1 : alarm enabled Bit8 = 0 : alarm disabled |
| 6 | ALID | U4 | 4 | alarm ID |
| 7 | ALTX | A | 80 | alarm text |
| 8 | CEED | BOOLEAN | 1 | True : valid False : not valid |

| No | Name | Format | Size | Explanation |  |
| --- | --- | --- | --- | --- | --- |
| 9 | CEID | U2 |  | collection event ID |  |
| 10 | CEPACK | L,U1 |  | 0 : no error 1 : CPNAME doesn't exist 2 : the incorrect value is specified in CEPVAL 3 : the incorrect format is specified in CEPVAL 4 : the CPNAME usage isn't valid |  |
| 11 | CEPVAL | A,B,BOOLEAN, Fx,Ix,J,L,Ux |  | the command extension parameter value |  |
| 12 | COMMACK | B | 1 | 0 : ACK 1 : denied, retry 2～63 : hold |  |
| 13 | CPACK | B | 1 | 0 : no error 1 : CPNAME doesn't exist 2 : the incorrect value is specified in CPVAL 3 : the incorrect format is specified in CPVAL ≧4 : another equipment error |  |
| 14 | CPNAME | A | Max 40 | command parameter name |  |
| 15 | CPVAL | A,B,BOOLEAN, Fx,Ix,J,Ux |  | command parameter value |  |
| 16 | DATAID | U2 U4 | 4 | Data ID U2 for S2F49 |  |
| 17 | DRACK | B | 1 |  | 0 :ACK 1 : denied, insufficient space 2 : denied, incorrect format 3 : denied, at least one RPTID is already defined 4 : denied, at least one VID is already defined ≧5 : some other error |
| 18 | EAC | B | 1 |  | 0 :ACK 1 : denied, there isn't at least one constant 2 : denied, busy 3 : denied, at least constant is beyond the allowed range ≧4 : some other error |
| 19 | ECDEF | A,B,BOOLEAN, Fx,Ix,J,Ux |  |  | equipment constant default value |

| No | Name | Format | Size | Explanation |  |
| --- | --- | --- | --- | --- | --- |
| 20 | ECID | U2 | 2 |  | equipment constant ID |
| 21 | ECMAX | A,B,BOOLEAN, Fx,Ix,J,Ux |  |  | equipment constant maximum value |
| 22 | ECMIN | A,B,BOOLEAN, Fx,Ix,J,Ux |  |  | equipment constant minimum value |
| 23 | ECNAME | A |  |  | equipment constant name |
| 24 | ECV | A,B,BOOLEAN, Fx,Ix,J,Ux |  |  | equipment constant |
| 25 | ERACK | B | 1 |  | 0 :ACK 1 : denied, there isn't at least one CEID ≧2 :some other error |
| 26 | HCACK | B | 1 |  | 0 : confirmed, the command was executed 1 : command doesn't exist 2 : currently not able to execute 3 : at least one parameter isn't valid 4 :confirmed, the command will be executed and completion will be notified by an event 5 : rejected, already requested 6 : object doesn't exist 7 : NG (Used when a single port receives multiple transfer commands for input.) 8～63:hold |
| 27 | LRACK | B | 1 | 0 :ACK 1 : denied, insufficient space 2 : denied, incorrect format 3 : denied, at least one CEID link is already defined 4 : denied, there isn't at least one CEID 5 : denied, there isn't at least one RPTID ≧6: some other error |  |
| 28 | MDLN | A | Max6 | equipment format |  |
| 29 | MEXP | A |  | Sxx,Fyy |  |
| 30 | MHEAD | B |  | message header of the message that is in error |  |
| 31 | OBJSPEC | A |  | the text string used to indicate the specified object instance |  |
| 32 | OFLACK | B | 1 | 0 : offline confirmed 1～63 :hold |  |
| 33 | ONLACK | B | 1 | 0 : online confirmed 1 : not allowed to go online 2 : already online 3～63 :hold |  |

| No | Name | Format | Size | Explanation |
| --- | --- | --- | --- | --- |
| 34 | RCMD | A |  | remote command or command sequence |
| 35 | RPTID | U2 | 2 | report ID |
| 36 | SHEAD | B | 1 | message headers related to the transaction timer |
| 37 | SOFTREV | A | Max6 | software revision code |
| 38 | SV | A,B,BOOLEAN, Fx,Ix,J,L,Ux |  | status variable data |
| 39 | SVID | U2 | 2 | status variable ID |
| 40 | SVNAME | A |  | status variable name |
| 41 | TEXT | A |  | one line of text |
| 42 | TIACK | B | 1 | time confirmation code 0 :OK 1 : error, not okayed 2～63 :hold |
| 43 | TID | B | 1 | 0 : single or main terminal 1 : additional terminal of the same equipment |
| 44 | TIME | A | 16 | YYYYMMDDhhmmsscc |
| 45 | UNIT | A |  | those allowed by section 9 of SEMI E5 |
| 46 | V | A,B,BOOLEAN, Fx,Ix,J,L,Ux |  | variable data |
| 47 | VID | U2 | 2 | variable ID |

Table of Variables

This section explains the NPC variables and VIDs.

VID Numbering

The following chart summarizes how VIDs are assigned to NPC variables.

Table 6	- VID Numbering

| No | 所属 | VARIABLE TYPE | VID |
| --- | --- | --- | --- |
| 1 | Daifuku defined variables VID 1～300 | GEM defined variables | 1～50 |
| 2 |  | SEM defined variables | 51～200 |
| 3 |  | Daifuku defined variables | 201～300 |
| 4 | Supplier defined VIDs 301～ | Supplier defined VIDs | 301～ |

Note）

All variables defined by GEM or SEM that are not included in this specification that are specified by the supplier are assigned a VID of 301 or above.

NPC variables

The following chart lists the variables used by the NPC.

< Data in the chart >

VID: The variable's VID.

Variable Name: The name of the variable.

Class: The class of the variable's data.

Format: The format of the variable's data.

Explanation: 	An explanation of the variable.

*SV whose back ground color is gray,is not used in this project.

[Japanese]

*背景がGrayのSVは使用しない。

Table 7	- Variables used by the NPC

| VID | Variable Name | Class | Explanation |
| --- | --- | --- | --- |
|  |  | Format |  |
| 1 | AlarmID | DVVAL | Alarm ID |
|  |  | <A[1-64]> |  |
| - | AlarmID | SV | Alarm ID |
|  |  | ※refer to VID 1. |  |
| 2 | AlarmText | DVVAL | Alamr Text |
|  |  | <A[1..80]> |  |
| - | AlarmText | SV | Alarm Text |
|  |  | ※refer to VID 2 |  |
| 3 | EnhancedUnitAlarms | SV | Current alarms of all units. |
|  |  | <L[n] <EnhancedUnitAlarmInfo> : : > |  |
| 4 | EnhancedUnitAlarmInfo | SV | Current alarm information associated wits a particular unit. [PortError] UnitID=PortID [NPCError] UnitID=EQID |
|  |  | <L[3] <UnitID> <AlarmID> <AlarmText> > |  |
| 8 | UnitID | DVVAL | ID of the Unit |
|  |  | <A[1-64]> |  |
| - | UnitID | SV | ID of the Unit |
|  |  | ※refer to VID 8. |  |
| 9 | PortID | DVVAL | unique ID of the portname |
|  |  | <A[1..64]> |  |
| - | PortID | SV | unique ID of the portname |
|  |  | ※Refer to VID 9. |  |
| 10 | PortState | DVVAL | Port Transfer state 1 : OutOfService 2 : PortInService |
|  |  | U2 |  |
| - | PortState | SV | Port Transfer state 1 : OutOfService 2 : PortInService |
|  |  | ※refer to VID 10. |  |
| 11 | EnhancedPortState | SV | Current States of all the ports. |
|  |  | <L[n] <EnhancedPortInfo> : : > |  |
| 12 | EnhancedPortInfo | SV | Port information associated with a particular port. |
|  |  | <L[2] <PortID> <PortState> > |  |

SECS MESSAGES

This chapter explains the stream and function of the SECS messages used by the NPC.

Message List

The following chart lists the minimum SECS messages supported by the NPC.

< Data in the chart >

SECS: The message’s stream and function.

SEND: “H” indicates the message is sent from the host to the NPC. 
“E” indicates that the message is sent from the NPC to the
 host.

Format: The format indicated in SEMI E5’s message definition.

Data Example An example of the message data in SML※1 format.

Note: ”W” in the chart indicates Wait Bit (waiting for a secondary message).

※１　SMLI is registered trademark, and SECS Message Language (tn) is a trademark of GW Associates, Inc.

Table 8	- SECS Messages Used by the NPC

| SECS | SEND | Format | Data Example |  |
| --- | --- | --- | --- | --- |
| S1F1 | H,E | S1F1 W . /* header only */ | S1F1 W . |  |
| S1F2 | H | S1F2 <L[n]>. | S1F2 <L[n]>. |  |
|  | E | S1F2 <L[2] <A MDLN> <A SOFTREV> >. | S1F2 <L[2] <A ‘AMHS’> <A ‘VER1.0’> >. |  |
| S1F3 | H | S1F3 W <L[n] <Ux SVID> : >. | S1F3 W <L[2] <U2 4> <U2 11> >. |  |
| S1F4 | E | S1F4 <L[n] <SV> : >. | S1F4 <L[2] <> <> >. |  |
| S1F13 | H | S1F13 W <L[n]>. | S1F13 W <L[n]>. |  |
|  | E | S1F13 W <L[2] <A MDLN> <A SOFTREV> >. | S1F13 W <L[2] <A ‘AMHS’> <A ‘VER1.0’> >. |  |
| S1F14 | H | S1F14 <L[2] <B COMMACK> <L[n] : > >. | S1F14 <L[2] <B 0x00> <L[0] > >. |  |
|  | E | S1F14 <L[2] <B COMMACK> <L[2] <A MDLN> <A SOFTREV> > >. | S1F14 <L[2] <B 0x00> <L[2] <A ‘AMHS’> <A ‘VER1.0’> > >. |  |
| S1F17 | H | S1F17 W . /* header only */ | S1F17 W . |  |
| S1F18 | E | S1F18 <B ONLACK>. | S1F18 <B 0x00> |  |
| S2F31 | E | S2F31 W <A TIME>. | S2F31 W <A ‘2010123123565901’>. ※(2010/12/31 23:59:59.01) |  |
| S2F32 | H | S2F32 <B TIACK>. | S2F32 <B 0x00>. |  |
| S2F33 | H | S2F33 W <L[2] <Ux DATAID> <L[n] <Ux RPTID> <L[n] <Ux VID> : > > : > >. |  | S2F33 W <L[2] <U4 0> /* DATAID */ <L[2] <L[2] <U2 1> /* RPTID */ <L[1] <U2 62> /* VID */ > > <L[2] <U2 2> /* RPTID */ <L[2] <U2 62> /* VID */ <U2 74> /* VID */ > > > >. |
| S2F34 | E | S2F34 <B DRACK>. |  | S2F34 <B 0x00>. |
| S2F35 | H | S2F35 W <L[2] <Ux DATAID> <L[n] <Ux CEID> <L[n] <Ux RPTID> : > > : > >. |  | S2F35 W <L[2] <U4 0> /* DATAID */ <L[2] <L[2] <U2 3> /* CEID */ <L[1] <U2 1> /* RPTID */ > > <L[2] <U2 108> /* CEID */ <L[1] <U2 1> /* RPTID */ > > > >. |
| S2F36 | E | S2F36 <B LRACK>. |  | S2F36 <B 0x00>. |
| S2F37 | H | S2F37 W <L[2] <BOOLEAN CEED> <L[n] <Ux CEID> : > >. |  | S2F37 W <L[2] <BOOLEAN T> <L[2] <U2 3> /* CEID */ <U2 108> /* CEID */ > >. |
| S2F38 | E | S2F38 <B ERACK>. |  | S2F38 <B 0x00>. |
| S5F1 | E | S5F1 W <L[3] <B ALCD> <Ux ALID> <A ALTX> >. |  | S5F1 W <L[3] <B 0x80> <U4 7799057> <A ‘Emergency Stop Error’> >. |
| S5F2 | H | S5F2 <B ACKC5>. |  | S5F2 <B 0x00>. |
| S6F11 | E | S6F11 W <L[3] <Ux DATAID> <Ux CEID> <L[n] <L[2] <Ux RPTID> <L[n] <VID> : > > > >. |  | S6F11 W <L[3] <U4 0> <U2 3> <L[1] <L[2] <U2 1> <L[1] <A ‘AMHS001’> > > > >. |
| S6F12 | H | S6F12 <B ACKC6>. |  | S6F12 <B 0x00>. |

EVENTS

The NPC supports the event reports based on the “Dynamic Event Report Setting Modifications” included in GEM. This chapter explains the events used by the NPC.

EVENT and EVENT REPORT

CEID Numbering

The following chart summarizes how CEIDs are assigned to NPC events.

Table 9	- CEID Numbering

| No | 所属 | EVENT TYPE | CEID |
| --- | --- | --- | --- |
| 1 | CEIDs defined in Daifuku specifications 1～700 | GEM defined events | 1～50 |
| 2 |  | SEM defined events | 51～500 |
| 3 |  | Daifuku defined events | 501～700 |
| 4 | supplier defined CEIDs 701～ | supplier defined CEIDs | 701～ |

Note:

All events defined by GEM or SEM that are not included in this specification that are specified by the supplier are assigned a CEID of 701 or above.

NPC Events

This section explains the types of events used by the NPC.

Each event report should be sent when any corresponding event occurs.

No specific order of event reports is determined.  For your reference, basic scenarios are indicated as below:

- Port Status Transition Events

Table 11	- Port Status Transition Events

| CEID | EVNET NAME | OLD STATUS | NEW STATUS | REPORT ID |
| --- | --- | --- | --- | --- |
| 260 | PortOutOfService | IN SERVICE | OUT OF SERVICE | 2 |
| 261 | PortInService | OUT OF SERVICE | IN SERVICE | 2 |

- Alarm Related Events

Table 12	- Alarm Related Events

| CEID | EVNET NAME | OLD STATUS | NEW STATUS | REPORT ID |
| --- | --- | --- | --- | --- |
| 301 | UnitAlarmCleared | ALARM | NO ALARM | 3 |
| 302 | UnitAlarmSet | NO ALARM¦ALARM | ALARM | 3 |

The report condition of UnitAlarm

・UnitAlarmSet must be pair with UnitAlarmClear each alarmID and UnitID. (Except for DFK)

・AlarmID which may be simultaneously generated in the same Unit is taken as to one. （DFK only）

Example: When AlarmID: 4321 occur after AlarmID: 1234 occurred at Unit Crane01, report UnitAlarmSet of 4321 after reporting UnitAlarmCleared of 1234.

[Japanese]

UnitAlarmの報告条件

・UnitAlarmSetとUnitAlarmClearedは必ずAlarmID, UnitIDで対になっていること（DFKは除く）

・同一Unitで同時に発生しうるAlarmIDは一つまでとする (DFKのみ)

例：Unit　Crane01でAlarmID：1234が発生した後にAlarmID：4321が発生した場合、1234のUnitAlarmClearedを報告してから、4321のUnitAlarmSetを報告すること。

NPC Event Reports

The following chart reveals the report format (default) the host uses in regards to the NPC as an example of a dynamic event report:

The host is able to define the format of event report by S2F33.

When an event report format is defined, it is not necessary to comply with the report format in this section.

※Report format in the following chart only indicates reporting section of an event report by S6F11.

Table 13	- Basic Event Report Format

| RPTID | VARIABLE（VID） | REPORT FORMAT |
| --- | --- | --- |
| 1 |  | <L[0] > |
| 2 | PortID | <L[1] <A $PortID> > |
| 2 | UnitID AlarmID AlarmText | <L[3] <A $UnitID> <U4 $AlarmID> <A $AlarmText> > |

SCENARIO

This chapter describes various scenarios.

COMMON SCENARIO

This section describes basic scenarios for NPC.

Each event in the scenarios occurs when a corresponding event occurs.  The following scenario is just an example and does not have to be fully complied.

ACTIVATION FROM THE AMHS
 (from EQUIPMENT OFFLINE)

8.1.1. ACTIVATION FROM THE AMHS (EQUIPMENT OFFLINE)

AMHS INFORMATION

HSMS

Communication Status

Control Status

HSMS

Communication Status

Control Status

NOT CONNECTED

NONE

NONE

Op: toggled to ENABLE

NOT CONNECTED

DiSABLE

EQ OFFLINE

Connect(exanple)

NOT SELECTED

accept(example)

NOT COMMUNICATED

NOT SELECTED

SELECTED

SELECTED

Select.req

COMMUNICATING

Select.rsp

COMMUNICATINOG

S1F13 Establish Communication Requiest

S1F14 Establish Communication Requiest ACK

Case: ONLACK=2

S1F17 Online Request

S1F18 Online Request ACK

ONLINE REMOTE

S2F31 Date and Time Set Request

S2F32 Date and Time Set ACK

S2F33 Define Report

S2F34 Define Report ACK

S2F33 Define Report

S2F34 Define Report ACK

S2F35 Link Event Report

S2F36 Link Event Report ACK

S2F37 Enable/Disable Event Report

S2F38 Enable/Disable Event Report ACK

S2F37 Enable/Disable Event Report

S2F38 Enable/Disable Event Report ACK

S1F3 Specified Equipment Status Request

All data synchronization

S1F4 Specified Equipment Status Data

ONLINE REMOTE

HOST

SC

COMMENTS

S1F13 Establish Communication Requiest

S1F14 Establish Communication Requiest ACK

Figure 3	- ACTIVATION FROM THE AMHS (EQUIPMENT OFFLINE)

ACTIVATION FROM THE HOST (ONLINE REMOTE)

8.1.1. ACTIVATION FROM THE AMHS (EQUIPMENT ONLINE)

AMHS INFORMATION

HSMS

Communication Status

Control Status

HSMS

Communication Status

Control Status

NOT CONNECTED

NONE

NONE

Op: toggled to ENABLE

NOT CONNECTED

DiSABLE

EQ OFFLINE

Connect(exanple)

NOT SELECTED

accept(example)

NOT COMMUNICATED

NOT SELECTED

SELECTED

SELECTED

Select.req

COMMUNICATING

Select.rsp

COMMUNICATINOG

S1F13 Establish Communication Requiest

S1F14 Establish Communication Requiest ACK

Case: ONLACK=2

S1F17 Online Request

S1F18 Online Request ACK

ONLINE REMOTE

S2F31 Date and Time Set Request

S2F32 Date and Time Set ACK

S2F33 Define Report

S2F34 Define Report ACK

S2F33 Define Report

S2F34 Define Report ACK

S2F35 Link Event Report

S2F36 Link Event Report ACK

S2F37 Enable/Disable Event Report

S2F38 Enable/Disable Event Report ACK

S2F37 Enable/Disable Event Report

S2F38 Enable/Disable Event Report ACK

S1F3 Specified Equipment Status Request

All data synchronization

S1F4 Specified Equipment Status Data

ONLINE REMOTE

HOST

SC

COMMENTS

S1F13 Establish Communication Requiest

S1F14 Establish Communication Requiest ACK

Figure 4	- ACTIVATION FROM THE HOST (ONLINE REMOTE)

NPC SCENARIO

This section describes basic scenarios for NPC.

Each event in the scenarios occurs when a corresponding event occurs.

PURGE UNIT ERROR OCCURS

PURGE UNIT ERROR RELEASE

Appendix

Using VID when synchronize

NPC

| VID | Variable Name |
| --- | --- |
| 3 | EnhancedUnitAlarms |
| 11 | EnhancedPortState |

Revision History

| Date | Full Version | Description | Prepared by | Approved by |
| --- | --- | --- | --- | --- |
| 2021/09/6 | 1.0.0 | First edition | T.Sanada | H.Nakagawa |

| NPC COMMUNICATION SPECIFICATION |  |  |
| --- | --- | --- |
|  |  | Full Version1.0.0 |
|  |  | Issued on 2024年9月11日 |
| Supervised by |  |  |
| Written by | T.Sanada |  |
| Edited by | - |  |
| Issued by | DAIFUKU CO. LTD., |  |
|  | CIM Group, System Department, Production Operations, Cleanroom Division |  |
| Copyright Ⓒ | 2018 - 2024 DAIFUKU CO. LTD., |  |
|  | 2 |  |